# Textile specific REACH restrictions

In this document we list textile specific restrictions of substances that are listed in Annex XVII of the REACH regulation. We want to inform you that this list may not be complete and that the official REACH regulation is the only legally binding document. The entire Annex XVII with further substance specific information can be found on ECHA CHEM.

## COMMISSION REGULATION (EU) 2018/1513

Annex XVII to Regulation (EC) No 1907/2006 is amended as follows:

(1) The following entry is added:

| The substances listed in column 1 of the following table | 1. Shall not be placed on the market after 1 November 2020 in any of the following: (a) clothing or related accessories; (b) textiles other than clothing which, under normal or reasonably foreseeable conditions of use, come into contact with human skin to an extent similar to clothing; (c) footwear; if the clothing, related accessory, textile other than clothing or footwear is for use by consumers and the substance is present in a concentration, measured in homogeneous material, equal to or greater than that specified for that substance in Appendix 12. 2. By way of derogation, in relation to the placing on the market of formaldehyde [CAS No 50-00-0] in jackets, coats or upholstery, the relevant concentration for the purposes of paragraph 1 shall be 300 mg/kg during the period between 1 November 2020 and 1 November 2023. The concentration specified in Appendix 12 shall apply thereafter. 3. Paragraph 1 shall not apply to: (a) clothing, related accessories or footwear, or parts of clothing, related accessories or footwear, made exclusively of natural leather, fur or hide; (b) non-textile fasteners and non-textile decorative attachments; (c) second-hand clothing, related accessories, textiles other than clothing or footwear (d) wall-to-wall carpets and textile floor coverings for indoor use, rugs and runners. 4. Paragraph 1 shall not apply to clothing, related accessories, textiles other than clothing, or footwear within the scope of Regulation (EU) 2016/425 of the European Parliament and of the Council (*) or Regulation (EU) 2017/745 of the European Parliament and of the Council (**). 5. Paragraph 1(b) shall not apply to disposable textiles. ‘Disposable textiles’ means textiles that are designed to be used only once or for a limited time and are not intended for subsequent use for the same or a similar purpose. 6. Paragraphs 1 and 2 shall apply without prejudice to the application of any stricter restrictions set out in this Annex or in other applicable Union legislation. 7. The Commission shall review the exemption in paragraph 3(d) and, if appropriate, modify that point accordingly. (*) Regulation (EU) 2016/425 of the European Parliament and of the Council of of 9 March 2016 on personal protective equipment and repealing Council Directive 89/686/EEC (OJ L 81, 31.3.2016, p. 51). (**) Regulation (EU) 2017/745 of the European Parliament and of the Council of 5 April 2017 on medical devices, amending Directive 2001/83/EC, Regulation (EC) No 178/2002 and Regulation (EC) No 1223/2009 and repealing Council Directives 90/385/EEC and 93/42/EEC (OJ L 117, 5.5.2017, p. 1). |
| --- | --- |

(2) The following Appendix 12 is added: Entry 72 — restricted substances and maximum concentration limits by weight in homogeneous materials:

| Substances | Concentration limit by weight |
| --- | --- |
| Cadmium and its compounds (listed in Annex XVII, Entry 28, 29, 30, Appendices 1-6) | 1 mg/kg after extraction (expressed as Cd metal that can be extracted from the material) |
| Chromium VI compounds (listed in Annex XVII, Entry 28, 29, 30, Appendices 1-6) | 1 mg/kg after extraction (expressed as Cr VI that can be extracted from the material) |
| Arsenic compounds (listed in Annex XVII, Entry 28, 29, 30, Appendices 1-6) | 1 mg/kg after extraction (expressed as As metal that can be extracted from the material) |
| Lead and its compounds (listed in Annex XVII, Entry 28, 29, 30, Appendices 1-6) | 1 mg/kg after extraction (expressed as Pb metal that can be extracted from the material) |
| Benzene | 5 mg/kg |
| Benz[a]anthracene | 1 mg/kg |
| Benz[e]acephenanthrylene | 1 mg/kg |
| benzo[a]pyrene; benzo[def]chrysene | 1 mg/kg |
| Benzo[e]pyrene | 1 mg/kg |
| Benzo[j]fluoranthene | 1 mg/kg |
| Benzo[k]fluoranthene | 1 mg/kg |
| Chrysene | 1 mg/kg |
| Dibenz[a,h]anthracene | 1 mg/kg |
| α,α,α,4-tetrachlorotoluene; p-chloro benzotrichloride | 1 mg/kg |
| α,α,α-trichlorotoluene; benzotrichloride | 1 mg/kg |
| α-chlorotoluene; benzyl chloride | 1 mg/kg |
| Formaldehyde | 75 mg/kg |
| 1,2-benzenedicarboxylic acid; di-C 6-8-branched alkylesters, C 7-rich | 1 000 mg/kg (individually or in combination with other phthalates in this entry or in other entries of Annex XVII that are classified in Part 3 of Annex VI to Regulation (EC) No 1272/2008 in any of the hazard classes carcinogenicity, germ cell mutagenicity or reproductive toxicity, category 1A or 1B |
| Bis(2-methoxyethyl) phthalate | 1 000 mg/kg (individually or in combination with other phthalates in this entry or in other entries of Annex XVII that are classified in Part 3 of Annex VI to Regulation (EC) No 1272/2008 in any of the hazard classes carcinogenicity, germ cell mutagenicity or reproductive toxicity, category 1A or 1B |
| Diisopentylphthalate | 1 000 mg/kg (individually or in combination with other phthalates in this entry or in other entries of Annex XVII that are classified in Part 3 of Annex VI to Regulation (EC) No 1272/2008 in any of the hazard classes carcinogenicity, germ cell mutagenicity or reproductive toxicity, category 1A or 1B |
| Di-n-pentyl phthalate (DPP) | 1 000 mg/kg (individually or in combination with other phthalates in this entry or in other entries of Annex XVII that are classified in Part 3 of Annex VI to Regulation (EC) No 1272/2008 in any of the hazard classes carcinogenicity, germ cell mutagenicity or reproductive toxicity, category 1A or 1B |
| Di-n-hexyl phthalate (DnHP) | 1 000 mg/kg (individually or in combination with other phthalates in this entry or in other entries of Annex XVII that are classified in Part 3 of Annex VI to Regulation (EC) No 1272/2008 in any of the hazard classes carcinogenicity, germ cell mutagenicity or reproductive toxicity, category 1A or 1B |
| N-methyl-2-pyrrolidone; 1-methyl-2-pyr rolidone (NMP) | 3 000 mg/kg |
| N,N-dimethylacetamide (DMAC) | 3 000 mg/kg |
| N,N-dimethylformamide; dimethyl formamide (DMF) | 3 000 mg/kg |
| 1,4,5,8-tetraaminoanthraquinone; C.I. Dis perse Blue 1 | 50 mg/kg |
| Benzenamine, 4,4′-(4-iminocyclohexa- 2,5- dienylidenemethylene)dianiline hydrochloride; C.I. Basic Red 9 | 50 mg/kg |
| [4-[4,4′-bis(dimethylamino)benzhydrylidene]cyclohexa-2,5-dien-1-ylidene]dimethylammonium chloride; C.I. Basic Violet 3 with ≥ 0,1 % of Michler's ketone (EC no. 202-027-5) | 50 mg/kg |
| 4-chloro-o-toluidinium chloride | 30 mg/kg |
| 2-Naphthylammoniumacetate | 30 mg/kg |
| 4-methoxy-m-phenylene diammonium sulphate; 2,4-diaminoanisole sulphate | 30 mg/kg |
| 2,4,5-trimethylaniline hydrochloride | 30 mg/kg |
| Quinoline | 50 mg/kg |

## COMMISSION REGULATION (EU) 2016/26

In Annex XVII to Regulation (EC) No 1907/2006, the following new entry 46a is added:

| 46a. Nonylphenol ethoxylates (NPE) (C2H4O)nC15H24O | 1. Shall not be placed on the market after 3 February 2021 in textile articles which can reasonably be expected to be washed in water during their normal lifecycle, in concentrations equal to or greater than 0,01 % by weight of that textile article or of each part of the textile article. 2. Paragraph 1 shall not apply to the placing on the market of second-hand textile articles or of new textile articles produced, without the use of NPE, exclusively from recycled textiles. 3. For the purposes of paragraphs 1 and 2, “textile article” means any unfinished, semi-finished or finished product which is composed of at least 80 % textile fibres by weight, or any other product that contains a part which is composed of at least 80 % textile fibres by weight, including products such as clothing, accessories, interior textiles, fibres, yarn, fabrics and knitted panels. |
| --- | --- |

## COMMISSION REGULATION (EU) 2021/2030

In Annex XVII to Regulation (EC) No 1907/2006, the following entry is added:

| 76. N,N-dimethylformamide CAS No 68-12-2 EC. No 200-679-5 | 1. Shall not be placed on the market as a substance on its own, as a constituent of other substances, or in mixtures in a concentration equal to or greater than 0,3 % after 12 December 2023 unless manufacturers, importers and downstream users have included in the relevant chemical safety reports and safety data sheets, Derived No-Effect Levels (DNELs) relating to exposure of workers of 6 mg/m3 for exposure by inhalation and 1,1 mg/kg/day for dermal exposure. 2. Shall not be manufactured, or used, as a substance on its own, as a constituent of other substances, or in mixtures in a concentration equal to or greater than 0,3 % after 12 December 2023 unless manufacturers and downstream users take the appropriate risk management measures and provide the appropriate operational conditions to ensure that exposure of workers is below the DNELs specified in paragraph 1. 3. By way of derogation from paragraphs 1 and 2, the obligations laid down therein shall apply from 12 December 2024 in relation to placing on the market for use, or use, as a solvent in direct or transfer polyurethane coating processes of textiles and paper material or the production of polyurethane membranes, and from 12 December 2025 in relation to placing on the market for use, or use, as a solvent in the dry and wet spinning processes of synthetic fibres. |
| --- | --- |

## COMMISSION REGULATION (EU) 2024/2462

In Annex XVII to Regulation (EC) No 1907/2006, the following entry is added:

| 79. Undecafluorohexanoic acid (PFHxA), its salts and PFHxA-related substances: (a) having a linear or branched perfluoropentyl group with the formula C5F11 - directly attached to another carbon atom as one of the structural elements; or (b) having a linear or branched perfluorohexyl group with the formula C6F13-. The following substances are excluded from this designation: (a) C6F14; (b) C6F13-C(=O)OH, C6F13-C(=O)O-X′ or C6F13-CF2-X′ (where X′ = any group, including salts); (c) any substance having a perfluoroalkyl group C6F13- directly attached to an oxygen atom at one of the non-terminal carbon atoms. | 1. Shall not, from 10 October 2026 be placed on the market, or used, in a concentration equal to or greater than 25 ppb for the sum of PFHxA and its salts, or 1000 ppb for the sum of PFHxA-related substances, measured in homogeneous material, in the following: (a) textiles, leather, furs and hides in clothing and related accessories for the general public; (b) footwear for the general public; (c) paper and cardboard used as food contact materials within the scope of Regulation (EC) No 1935/2004; (d) mixtures for the general public; (e) cosmetic products as defined in Article 2(1), point (a), of Regulation (EC) No 1223/2009. 2. Shall not, from 10 October 2027 be placed on the market, or used, in a concentration equal to or greater than 25 ppb for the sum of PFHxA and its salts, or 1000 ppb for the sum of PFHxA-related substances, measured in homogeneous material, in textiles, leather, furs and hides, other than in clothing and related accessories referred to in paragraph 1, for the general public. 3. Paragraphs 1 and 2 shall not apply to the following: (a) personal protective equipment intended to protect users against risks within the scope of risk category III, points (a), (c) to (f), (h), and (l) of Annex I to Regulation (EU) 2016/425; (b) devices within the scope of Regulation (EU) 2017/745; (c) devices within the scope of Regulation (EU) 2017/746; (d) textiles used as construction textiles. 4. Shall not, from 10 April 2026 be placed on the market, or used, in a concentration equal to or greater than 25 ppb for the sum of PFHxA and its salts, or 1000 ppb for the sum of PFHxA-related substances, in: (a) firefighting foams and firefighting foam concentrates for training and for testing, except functional testing of the firefighting systems provided that all releases are contained; (b) firefighting foams and firefighting foam concentrates for public fire services, except where those services intervene at industrial fires at establishments covered by Directive 2012/18/EU of the European Parliament and of the Council (*) and they use the foams and the equipment for that purpose only. 5. Shall not, from 10 October 2029 be placed on the market, or used, in firefighting foams and firefighting foam concentrates for civil aviation (including in civilian airports) in a concentration equal to or greater than 25 ppb for the sum of PFHxA and its salts, or 1000 ppb for the sum of PFHxA-related substances. 6. Paragraphs 1, 2, 4 and 5 shall not apply to substances having a perfluoroalkyl group C6F13- directly attached to a sulphur atom that are prohibited in Annex I to Regulation (EU) 2019/1021 of the European Parliament and of the Council (**). 7. By way of derogation from paragraph 1, that paragraph shall not apply to articles and mixtures which were placed on the market before 10 October 2026. 8. By way of derogation from paragraph 2, that paragraph shall not apply to articles which were placed on the market before 10 October 2027. 9. For the purposes of this entry, PFHxA-related substances are substances that, based on their molecular structure, are considered to have the potential to degrade or be transformed to PFHxA. 6. Paragraphs 1, 2, 4 and 5 shall not apply to substances having a perfluoroalkyl group C6F13- directly attached to a sulphur atom that are prohibited in Annex I to Regulation (EU) 2019/1021 of the European Parliament and of the Council (**). 7. By way of derogation from paragraph 1, that paragraph shall not apply to articles and mixtures which were placed on the market before 10 October 2026. 8. By way of derogation from paragraph 2, that paragraph shall not apply to articles which were placed on the market before 10 October 2027. 9. For the purposes of this entry, PFHxA-related substances are substances that, based on their molecular structure, are considered to have the potential to degrade or be transformed to PFHxA. (*) Directive 2012/18/EU of the European Parliament and of the Council of 4 July 2012 on the control of major-accident hazards involving dangerous substances, amending and subsequently repealing Council Directive 96/82/EC (OJ L 197, 24.7.2012, p. 1, ELI: http://data.europa.eu/eli/dir/2012/18/oj). (**) Regulation (EU) 2019/1021 of the European Parliament and of the Council of 20 June 2019 on persistent organic pollutants (OJ L 169, 25.6.2019, p. 45, ELI: http://data.europa.eu/eli/reg/2019/1021/oj). |
| --- | --- |